| Experiment no.: | Date of Performance: |
| --- | --- |

Title: To Simulate Binary phase shift keying modulation and demodulation

Objectives: 1) To understand generation of BPSK signal

2) To demodulate BPSK signal

Software Required: Octave Online

Theory:

### Algorithm: BPSK Modulation and Demodulation

#### Step 1: Initialize parameters

- Set bit duration T=1 second.

- Define the input bit stream b=[1 0 1 0 1]

- Set the peak voltage Vp=1V.

- Define an empty array NRZ_out to store the encoded waveform.

#### Step 2: NRZ (Non-Return-to-Zero) Line Coding

- For each bit in the bit stream:

  - If the bit = 1, append 200 samples of +Vp+V_p+Vp​ to NRZ_out.

  - If the bit = 0, append 200 samples of −Vp-V_p−Vp​ to NRZ_out.

- The result is a bipolar NRZ waveform representing the bit sequence.

#### Step 3: Plot the bit impulses and NRZ waveform

- Plot the original bits using a stem plot.

- Plot the continuous NRZ waveform using a line plot.

#### Step 4: Generate the carrier signal

- Define the carrier frequency f=5 Hz

- Define a time vector t ranging from 0.005 to 5 seconds with step size 0.005.

#### Step 5: BPSK Modulation

- Multiply the NRZ waveform with the carrier signal:

Modulated=NRZ_out×( √2)Tcos(2πft)

This multiplication shifts the NRZ signal to a higher frequency — generating a BPSK modulated signal.

- Plot the modulated signal.

#### Step 6: Demodulation (Coherent Detection)

- Multiply the received (modulated) signal again by the same carrier:

demodulated=Modulated×(√2)Tcos

- Integrate the demodulated signal over each bit duration T using the trapezoidal rule (trapz function) to simulate the action of a correlator receiver.

#### Step 7: Decision and Bit Recovery

- For each integrated value:

  - If the result > 0, decide the received bit = 1.

  - Else, decide the received bit = 0.

- Store the detected bits in the array received.

#### Step 8: Plot the received bit impulses

- Use a stem plot to show the recovered bits after demodulation.

Conclusion:

| Name of Teaching Staff: | Signature |
| --- | --- |