| Experiment no.: | Date of Performance: |
| --- | --- |

Title: Simulation study of Performance of M-ary PSK .

Objectives: 1) To understand signal space representation of MPSK

2) To plot performance curve of MPSK

Software Required: Octave Online

### Algorithm: M-ary PSK (MPSK) Modulation and BER Performance

#### Step 1: Initialize Parameters

- Set modulation order M=16.

- Set the number of symbols to be transmitted:

num_symbol=1000

- Generate random integer data symbols:

data=randi([0,M−1], num_symbol,1 )

#### Step 2: MPSK Modulation

- Modulate the data using M-PSK modulation scheme:

txSig=pskmod(data, M, π/M)

(where π/M\pi/Mπ/M is the phase offset for better constellation symmetry)

- Display the transmitted constellation using a scatter plot.

#### Step 3: Transmit Signal through AWGN Channel

- Add Additive White Gaussian Noise (AWGN) to the transmitted signal with 20 dB SNR:

rxSig=awgn(txSig, 20)

- Display the noisy constellation using a scatter plot.

#### Step 4: Initialize BER Calculation Arrays

- Initialize empty arrays:

  - snr → to store SNR values (1 dB to 50 dB)

  - BER_sim → to store simulated Bit Error Rate

  - ber_theoretical → to store theoretical BER values

#### Step 5: Simulation Loop for BER Calculation

For each SNR value i=1 to 50dB:

- Set SNR:

snr(i)=I

- Convert to linear scale:

Eb/N0=10(i/10)

- Add noise to the transmitted signal:

rxSig=awgn(txSig, Eb/N0)

Estimate simulated BER:

  - Compute number of bit errors between transmitted and received signal amplitudes (approximation):

BER_sim(i)=biterr(int16(abs(txSig)), int16(abs(rxSig)))/num_symbol

- Compute theoretical BER for MPSK:

ber_theoretical(i) = erfc(Eb/N0⋅sin(π/M))

#### Step 6: Plot BER vs. SNR

- Plot the simulated BER and theoretical BER curves versus SNR.

- Add a legend to compare both results.

#### Step 7: End of Algorithm

Conclusion:

| Name of Teaching Staff: | Signature |
| --- | --- |