| Experiment no.: | Date of Performance: |
| --- | --- |

Title: Simulation study of Performance of M-ary QAM.

Objectives: 1) To understand signal space representation of QAM

2) To plot performance curve of QAM

Software Required: Octave Online

### Algorithm: M-ary QAM Modulation and BER Performance Simulation

#### Step 1: Initialize Parameters

- Set modulation order:

M=16

(This corresponds to 16-QAM, which transmits 4 bits per symbol since log2(16) =4 Set number of symbols to transmit:

num_symbol=1000

- Generate random integer data symbols:

data=randi([0,M−1],num_symbol,1)

#### Step 2: QAM Modulation

- Modulate the data using M-QAM scheme:

txSig=qammod(data, M) Display the transmitted constellation using a scatter plot.

#### Step 3: Add AWGN Noise

- Pass the transmitted signal through an Additive White Gaussian Noise (AWGN) channel with a fixed SNR of 20 dB:

rxSig=awgn(txSig, 20) Display the noisy constellation using a scatter plot.

#### Step 4: Initialize BER Arrays

- Initialize empty arrays:

  - snr → to store signal-to-noise ratio (SNR) values from 1 to 50 dB

  - BER_sim → to store simulated Bit Error Rates

  - ber_theoretical → to store theoretical Bit Error Rates

#### Step 5: Loop for BER Computation

For each SNR value i= 1to 50dB:

- Set SNR value:

snr(i)=I

- Convert to linear scale:

Eb/N0=10(i/10)

- Add corresponding noise:

rxSig=awgn(txSig, Eb/N0)

- Compute simulated BER:
Estimate bit errors between transmitted and received signal amplitudes:

BER_sim(i)=biterr(int16(abs(txSig)), int16(abs(rxSig)))num_symbol

- Compute theoretical BER for 16-QAM:
Using the approximate formula for square QAM:

ber_theoretical(i)=2×erfc(√0.4×Eb/N0)

#### Step 6: Plot BER vs SNR

- Plot both simulated BER and theoretical BER on the same graph.

- Label axes and include a legend:

  - X-axis → SNR (dB)

  - Y-axis → Bit Error Rate (BER)

#### Step 7: End of Algorithm

Conclusion:

| Name of Teaching Staff: | Signature |
| --- | --- |